# The Ford FE

Published October 2, 2026

Topics: Automobiles, Ford, Engines, Motor Racing

Ford's FE V-8 arrived for 1958 as a modern replacement for the aging

Y-block. It began as a smooth, torquey engine for full-sized cars and

Thunderbirds, but within a few years it had become the centre of Ford's

“Total Performance” campaign. FE engines powered family sedans, police

cars, pickups, luxury Mercurys, drag-racing Fairlane Thunderbolts,

NASCAR stock cars, Cobras, and the Ford GT40s that defeated Ferrari at

Le Mans.

The family ranged from 332 to 428 cubic inches in regular Ford

production. It included ordinary two-barrel engines, high-compression

four-barrel versions, triple two-barrels, dual four-barrels, low-,

medium-, and high-riser cylinder heads, cross-bolted main bearings, top-

and side-oiling systems, Tunnel Port heads, and the extraordinary

single-overhead-cam 427 “Cammer.” At the end came the 428 Cobra Jet, an

engine that combined relatively inexpensive production pieces into one

of the muscle era's most effective street and drag-racing packages.

The FE story is therefore not simply a march toward greater

displacement. It is the story of Ford learning that a big engine had to

breathe, survive, fit the car, meet racing rules, and remain affordable

enough to build.

## What “FE” means

The FE was Ford's medium-size big-block, physically larger and

heavier than the later Windsor small-block but smaller than the MEL

engines used by Mercury, Edsel, and Lincoln. “FE” is often expanded as

“Ford-Edsel,” although surviving Ford documentation does not establish

that interpretation beyond doubt. Whatever the initials originally

meant, they identify a distinct engine family built from 1958 through

1976.

All production FEs were 90-degree, pushrod, overhead-valve V-8s with

4.630-inch bore spacing. Their most recognizable feature is the

unusually wide intake manifold. Unlike most American V-8s, the FE's

intake extends beneath the valve covers and forms part of the upper

cylinder-head structure; the pushrods pass through openings in the

manifold. Removing an FE intake therefore exposes part of the

valvetrain, and the manifold itself is large and heavy.

The block used a deep skirt that extended below the crankshaft

centreline, providing a rigid crankcase. Most street versions had

two-bolt main-bearing caps. Competition 406s and 427s added cross-bolted

centre mains, with horizontal bolts passing through the block skirt into

the main caps to resist cap movement at high speed. Ford also developed

several oiling arrangements, culminating in the 427 side-oiler, which

directed oil to the main bearings before the camshaft and

valvetrain.

## The FE family dimensions

Version

Principal model years

Bore

Stroke

Nominal displacement

Basic purpose

332

1958–1959

4.000 in.

3.300 in.

332 cu. in./5.4 L

Original Ford FE

352

1958–1967

4.000 in.

3.500 in.

352 cu. in./5.8 L

Mainstream Ford, Mercury, and Thunderbird engine

361 Edsel

1958–1959

4.050 in.

3.500 in.

361 cu. in./5.9 L

Edsel E-400/Super Express

390

1961–1971

4.050 in.

3.785 in.

390 cu. in./6.4 L

High-volume passenger-car and light-truck FE

406

1962–1963

4.130 in.

3.785 in.

406 cu. in./6.7 L

Short-lived competition predecessor to the 427

427

1963–1968

4.232 in.

3.785 in.

426 cu. in./7.0 L actual

Racing-oriented, large-bore FE

410

1966–1967

4.050 in.

3.984 in.

410 cu. in./6.7 L

Mercury torque engine using the 428 crankshaft

428

1966–1970

4.130 in.

3.984 in.

428 cu. in./7.0 L

Long-stroke, production-oriented performance FE

360 truck

1968–1976

4.050 in.

3.500 in.

361 cu. in./5.9 L actual

Durable F-Series light-truck engine

Ford's names conceal a delightful paradox. The “427” actually

displaced about 426 cubic inches, while the nominally larger “428”

displaced about 427.6. The 427's identity came from its huge 4.232-inch

bore and shorter 3.785-inch stroke, which allowed large valves and high

engine speed. The 428 used a smaller 4.130-inch bore and longer

3.984-inch stroke, emphasizing torque and easier mass production.

## Model-year progression at a

glance

Model years

Principal FE varieties

1958

332 2V and 4V; 352 4V; Edsel 361 4V

1959

332 and 352 in Ford and Thunderbird; 361 continued in Edsel

1960

352 in regular and 360-horsepower High Performance form

1961

352 continued; 390 introduced in 2V, 4V, and High Performance

forms

1962

352 and 390; high-performance 406 with one four-barrel or three

two-barrels

1963

352, 390, and final 406; 427 introduced with single- and

dual-four-barrel induction

1964

352, 390, and 427; high-riser 427 powered Fairlane Thunderbolts and

racing Galaxies

1965

352 and 390; 427 medium-riser and side-oiler development

1966

352 and 390; Mercury 410; regular 428 and 428 Police Interceptor;

427 competition engines

1967

352, 390, 410, 427, and 428; 390 GT reached Mustang and Cougar; 427

Tunnel Port appeared

1968

390 GT, 427, and 428; 428 Cobra Jet introduced; 360 and 390

emphasized in trucks

1969–1970

390; 428, Cobra Jet, and Super Cobra Jet; FE performance-car use

declined as the 385 series arrived

1971

Final passenger-car 390s; 360 and 390 continued in light trucks

1972–1976

Low-compression, two-barrel 360 and 390 truck engines; FE production

ended during 1976

As with the Small Block Ford, individual ratings varied by vehicle,

transmission, emissions jurisdiction, and engine code. The timeline

identifies the principal United States offerings rather than every

calibration.

## 1958–1959: the 332, 352, and

Edsel 361

The original 332 used a 4.000-inch bore and 3.300-inch stroke. A

representative two-barrel version had approximately 8.9:1 compression

and produced 240 gross horsepower; a 9.5:1 four-barrel version was rated

at 265 horsepower and about 360 pound-feet. Even the smallest FE

therefore began with substantially more torque than Ford's preceding

small V-8s.

The 352 arrived simultaneously, retaining the 4.000-inch bore but

increasing stroke to 3.500 inches. In 1958 four-barrel Interceptor and

Thunderbird versions used approximately 10.2:1 compression and were

rated at 300 gross horsepower and 395 pound-feet. Lower-compression,

two-barrel versions followed, establishing the 352 as a durable

mainstream engine in Fords, Mercurys, Thunderbirds, and, eventually,

F-Series trucks.

Edsel received its own 361, created by combining the 352's 3.500-inch

stroke with a 4.050-inch bore. The 1958 E-400 version used a four-barrel

carburetor and 10.5:1 compression. Ford rated it at 303 gross horsepower

and 400 pound-feet—the “400” in its marketing name referred to torque,

not displacement. Its 1959 successor retained the 303-horsepower rating

with lower compression in some specifications.

These engines established the FE as a smooth, high-torque powerplant

rather than a racing engine. That changed rapidly.

## 1960: the 352 High

Performance

Ford's first serious FE performance package was the 1960 352 High

Performance. It combined 10.6:1 compression with a solid-lifter

camshaft, an aluminium intake manifold, a large four-barrel carburetor,

revised cylinder heads, and header-style cast-iron exhaust manifolds.

Ford advertised 360 gross horsepower at 6,000 rpm and approximately 380

pound-feet at 3,400 rpm.

One horsepower per cubic inch was a potent marketing threshold in

1960. More important, the 352 HP established the formula Ford would

apply repeatedly: improve breathing, increase compression, strengthen

the valvetrain, reduce intake weight, and allow the engine to operate at

much higher speed. Its limitations also encouraged Ford to add

displacement.

## 1961–1971: the 390

becomes the everyday FE

The 390 combined the 4.050-inch bore first used by the Edsel 361 with

a new 3.785-inch stroke. It became the most common and versatile FE,

appearing in full-sized Fords and Mercurys, Thunderbirds, Fairlanes,

Mustangs, Cougars, pickups, and other vehicles.

Representative 390

Model years

Induction

Compression

Advertised power

Advertised torque

Regular 390 2V

Early 1960s

2-barrel

Approximately 9.5:1

About 265 gross hp

About 401 lb.-ft.

Regular 390 4V

Early-to-mid 1960s

4-barrel

Approximately 10.0:1

300–315 gross hp

About 427 lb.-ft.

390 High Performance

1961

4-barrel

10.5:1

375 gross hp

427 lb.-ft.

390 High Performance, tri-power

1961

Three 2-barrels

10.5:1

401 gross hp

Approximately 430 lb.-ft.

390 GT

1966–1968

4-barrel

Approximately 10.5:1

315–335 gross hp

Approximately 427 lb.-ft.

390 truck

1972–1976

2-barrel

Approximately 8.2:1

Application-dependent SAE net

ratings

Torque-oriented calibration

The 390 High Performance borrowed the 352 HP's solid lifters,

aluminium intake, freer exhaust manifolds, and aggressive camshaft. A

triple-two-barrel version reached an advertised 401 horsepower. Yet the

majority of 390s were hydraulic-lifter, two- or four-barrel engines

designed to move heavy cars with little effort.

The 390 GT carried FE performance into the intermediate and pony-car

market. In Fairlanes, Mustangs, and Cougars, it used a four-barrel

carburetor, higher compression, a performance camshaft, dual exhausts,

and application-specific exhaust manifolds. Ratings varied from roughly

315 to 335 gross horsepower. A 390 Mustang was nose-heavy compared with

a small-block car, and it was not the fastest FE available, but its

broad torque and accessibility made the big-block pony car a practical

production reality.

## 1962–1963:

the 406 introduces cross-bolted strength

Ford enlarged the 390's bore from 4.050 to 4.130 inches to create the

406. Stroke remained 3.785 inches, preserving the high-performance

preference for bore area and engine speed. The extra bore crowded the

limits of the FE casting and drove improvements in block

construction.

The 406 initially used conventional two-bolt main caps, but later

competition blocks introduced cross-bolted centre mains. Spacers fitted

between the caps and block skirt, and horizontal bolts locked the

assembly together. This helped prevent main-cap movement under NASCAR

and drag-racing loads and became a defining feature of the later

427.

The regular four-barrel 406 used approximately 11.4:1 compression and

was rated at 385 gross horsepower at 5,800 rpm and 444 pound-feet at

3,400 rpm. A triple-two-barrel version produced an advertised 405

horsepower. For 1963, compression reached approximately 12.1:1 in some

versions. The 406 lasted barely two model years because racing pressure

demanded still more bore and better durability.

## 1963–1968: the 427 is built

to race

The 427 enlarged bore to 4.232 inches while retaining the 3.785-inch

stroke. Its actual displacement was approximately 425.98 cubic inches,

but Ford called it a 427 to align it with contemporary racing limits.

Special blocks used thicker decks, carefully formed cylinder walls,

cross-bolted centre main caps, forged steel crankshafts in many

applications, solid lifters, and internal balancing.

Early engines were “top-oilers”: oil reached the camshaft and

valvetrain before the crankshaft main bearings. The famous side-oiler,

developed during the middle 1960s, ran a dedicated oil gallery along the

side of the block and supplied the crankshaft first. This was a racing

solution, intended to protect the main and connecting-rod bearings

during sustained high-speed operation.

## Low-, high-, and medium-riser

427s

The “riser” descriptions refer to the height and shape of the intake

ports and matching manifold, not the engine block itself.

• The low-riser offered the best packaging for

full-sized production cars. Representative 1963–1964 single-four-barrel

versions were rated near 410 gross horsepower, while dual-four-barrel

versions reached 425 horsepower and approximately 480 pound-feet.

• The high-riser raised and straightened the intake

path for maximum high-rpm airflow. With very high compression and dual

Holley four-barrels, racing output exceeded the nominal 425-horsepower

production rating substantially. Its tall intake required special hood

clearance and sacrificed street manners.

• The medium-riser, introduced after the high-riser,

provided a more effective balance of port height, airflow, velocity, and

package size. Single-four-barrel versions were commonly rated at 410

horsepower, and dual-four-barrel versions at 425 horsepower. It became

the most versatile 427 wedge-head combination.

The 1964 Fairlane Thunderbolt demonstrated the high-riser's purpose.

Ford installed a dual-four-barrel 427 in a radically lightened

intermediate body with fibreglass and aluminium panels, special

suspension, and numerous production compromises. Ford quoted 425

horsepower and 480 pound-feet, while performance calculations suggested

well over 500 actual horsepower. The Thunderbolt dominated NHRA Super

Stock and became one of the purest factory drag cars ever offered.

## The 427 Tunnel Port

The 427 Tunnel Port moved the pushrods out of the intake port's

way—at least aerodynamically. Enormous, nearly straight intake passages

ran directly toward large valves, while selected pushrods passed through

brass tubes inside the ports. Ford offered single- and dual-four-barrel

manifolds. The design was effective at the sustained engine speeds of

NASCAR and influenced the ill-fated 302 Tunnel Port discussed in the

companion Small Block Ford article.

Unlike the smaller 302 version, the 427 had enough displacement to

maintain useful mixture velocity through the huge ports. Tunnel Port FEs

succeeded in NASCAR, drag racing, and endurance competition, although

they remained specialized, expensive engines rather than ordinary street

powerplants.

## The 427 SOHC “Cammer”

Ford developed the single-overhead-cam 427 in approximately ninety

days as an answer to Chrysler's 426 Hemi. The Cammer retained a

specially modified FE side-oiler block but replaced the pushrod heads

with hemispherical combustion chambers and one camshaft per cylinder

head. A chain more than six feet long drove both camshafts. Ford offered

different cam timing for the left and right banks to compensate for

chain dynamics.

With 12.0:1 compression, the single-four-barrel version was rated at

616 gross horsepower at 7,000 rpm; the dual-four-barrel version at 657.

Neither figure was a normal passenger-car rating. The engine was created

for racing and sold through Ford's parts network rather than installed

on a regular assembly line.

NASCAR refused to accept the Cammer as an ordinary production engine

and proposed weight penalties that made it uncompetitive. It never

achieved its intended NASCAR showdown with the Chrysler Hemi, but it

became formidable in drag racing, especially when supercharged or

fuel-injected. The Cammer was the most exotic expression of the FE

architecture, even though almost everything above its block deck

departed from the ordinary FE.

## Le Mans, NASCAR, and the

Cobra

The 427's greatest international achievement came in the Ford GT40 Mk

II and Mk IV. The side-oiler had to survive twenty-four hours near

racing speed, so Ford emphasized oiling, durability, and moderate engine

speed rather than the Cammer's spectacular peak output. GT40s powered by

427 FEs finished first, second, and third at Le Mans in 1966. The Mk IV

won again in 1967 with Dan Gurney and A. J. Foyt.

The FE also powered NASCAR Galaxies and Fairlanes, the lightweight

drag-racing Galaxies, Thunderbolts, and the most famous big-block Shelby

Cobras. Carroll Shelby's roadsters used both 427s and, in some

production cars, less costly 428s. “427 Cobra” consequently describes a

model and legend more reliably than it identifies the exact engine in

every original car.

## 1966–1967: the Mercury 410

The 410 combined the 390's 4.050-inch bore with the 428's 3.984-inch

stroke. It was essentially a smaller-bore companion to the 428 and, like

the 428, used external balance. Mercury offered it during 1966 and 1967

in full-sized cars.

With a four-barrel carburetor and 10.5:1 compression, Ford rated the

410 at 330 gross horsepower at 4,600 rpm and approximately 444

pound-feet at only 2,800 rpm. That low torque peak explains the engine

better than its horsepower number: the 410 was meant to move a large

Mercury effortlessly, not to compete with the short-stroke 427 at 7,000

rpm.

## 1966–1970: the

428 makes seven litres practical

The 427's large bore required expensive casting control and left

little space between cylinders. Ford created the 428 by combining the

406's more manageable 4.130-inch bore with a new 3.984-inch-stroke

crankshaft. The result actually displaced about 427.6 cubic inches,

fractionally more than the racing “427.”

The ordinary 1966 428 used hydraulic lifters, a four-barrel

carburetor, approximately 10.5:1 compression, and a cast nodular-iron

crankshaft. Ford rated it at 345 gross horsepower and roughly 462

pound-feet. It powered the Galaxie 500 7-Litre, Thunderbird, Mercury

S-55, and other large cars. A solid-lifter 428 Police Interceptor was

rated at approximately 360 horsepower for 1966; later PI versions used

hydraulic lifters.

The 428 was not as naturally suited to prolonged high-rpm racing as

the internally balanced, short-stroke 427. It was, however, cheaper to

manufacture, easier to install in volume-production cars, and stronger

in the engine-speed range most street drivers actually used. Those

qualities made the Cobra Jet possible.

## 1968–1970: the 428 Cobra Jet

Ford created the 428 Cobra Jet after dealer and racer pressure

demonstrated that carefully chosen existing parts could transform the

ordinary 428. The production engine combined the 428 block and

crankshaft with freer-flowing cylinder heads related to the 427

low-riser/428 Police Interceptor design, a cast-iron four-barrel intake,

a 735-cfm Holley carburetor, a performance hydraulic camshaft, high-flow

exhaust manifolds, stronger valve springs, and approximately 10.6:1

compression.

Ford advertised 335 gross horsepower at approximately 5,200–5,600 rpm

and 440–445 pound-feet at 3,400 rpm. The horsepower figure was

conspicuously conservative; contemporary testing showed that Cobra Jet

Mustangs and Fairlanes could run with cars carrying much higher ratings.

The low official number also helped with insurance classifications and

racing weight factors.

The Cobra Jet made its public reputation at the 1968 NHRA

Winternationals, where specially prepared Mustangs dominated their Super

Stock class. Production availability spread through Mustangs, Fairlanes,

Torinos, Cougars, Cyclones, Montegos, and the Shelby GT500KR. Ram Air

versions used a functional hood scoop but retained the same basic

advertised output.

## Super Cobra Jet and Drag

Pack

Ordering a 3.91:1 or 4.30:1 axle ratio with the appropriate Drag Pack

equipment transformed the engine into the 428 Super Cobra Jet. The SCJ

retained the Cobra Jet's heads, induction, camshaft, compression ratio,

and deliberately unchanged 335-horsepower rating. Its differences were

aimed at survival: stronger Le Mans-style connecting rods, different

pistons, an engine oil cooler, and crankshaft balancing matched to the

heavier rotating assembly.

The distinction is important. A Super Cobra Jet was not advertised as

more powerful than a Cobra Jet; it was built to withstand repeated

high-rpm acceleration with steep axle gearing. In practice, preparation

and production variation meant individual engines differed, but the

factory's intent was durability rather than a separate horsepower

tier.

## 1968–1976: the 360

and 390 finish in trucks

The truck 360 used the 390's 4.050-inch bore and the 352's 3.500-inch

stroke, yielding approximately 360.7 cubic inches. This is dimensionally

the same displacement as the earlier Edsel 361, but the truck engine's

lower compression, mild camshaft, two-barrel carburetor, and heavy-duty

calibration gave it an entirely different purpose.

Ford rated the 1968–1971 360 at approximately 215 gross horsepower

and 375 pound-feet. After the change to SAE net ratings, published

output was approximately 196 horsepower in some 1972 specifications,

with later ratings varying by year and emissions package. The 390 truck

engine offered greater stroke and torque with externally similar

castings; measuring stroke is often the only reliable way to distinguish

a rebuilt 360 from a 390.

Lower compression, exhaust-gas-recirculation systems, retarded

ignition, lean carburetion, and other emissions-era changes gradually

reduced advertised output. The FE nevertheless remained valuable in

F-Series pickups because torque, durability, and an enormous existing

production base mattered more than high-rpm horsepower. Production ended

during the 1976 model year as Ford's 335-series 351M and 400, and the

385-series 460, assumed its truck duties.

## Carburetion—and

almost no production fuel injection

Unlike the Small Block Ford, the FE completed its production life

before electronic fuel injection became common. Regular production

engines used carburetors throughout: two-barrels for economy and torque,

four-barrels for performance, three two-barrels for early

high-performance packages, and dual four-barrels for the most serious

427s.

Ford, Holley, and Autolite supplied different carburetors over the

years. The induction system could define an engine's personality, but

carburetor count alone did not determine performance. Compression,

camshaft timing, port size and shape, valve area, exhaust flow, and

engine durability mattered just as much. A dual-quad high-riser 427 and

a two-barrel truck 360 shared the FE family name while differing in

almost every operating priority.

Mechanical fuel injection appeared on specialized drag-racing and

competition FEs, especially the 427 SOHC, but Ford did not sell a normal

assembly-line, fuel-injected FE passenger car or pickup. By the time

Ford adopted electronic fuel injection widely during the 1980s, FE

production had ended.

## Gross horsepower versus

net horsepower

Nearly every celebrated FE performance figure is an SAE gross rating.

The engine was tested without the complete production air cleaner,

exhaust system, cooling fan, alternator, power-steering pump, and other

accessories. Beginning in 1972, manufacturers reported SAE net output

with production intake, exhaust, and accessories in place.

The rating change coincided with genuine reductions in compression,

cam timing, ignition advance, and exhaust flow. Consequently, the

apparent fall between a 1960s passenger-car FE and a 1970s truck engine

reflects both real mechanical changes and a stricter measurement system.

A 335-gross-horsepower Cobra Jet cannot be compared directly with a

196-net-horsepower truck 360 merely by subtracting the two ratings.

Factory ratings also served purposes beyond engineering. Racing

classification, insurance, corporate policy, and marketing all

influenced the printed number. Ford's 335-horsepower rating for the 428

Cobra Jet is the best-known example: the engine's performance strongly

suggests that the number did not describe its full capability.

## FE versus FT

Ford developed the related FT family for medium- and heavy-duty

trucks. FT engines shared FE ancestry and general architecture, but they

used heavy-duty blocks, crankshafts with different snouts,

larger-displacement oil pumps, different front dress, governed

carburetion, and other commercial-service components. Displacements

included 330, 359, 361, 389, and 391 cubic inches.

An FT is therefore not simply an FE with a truck air cleaner, and the

presence of “361” or “391” on an industrial engine does not identify an

Edsel 361 or passenger-car 390. Some components interchange with

modification, which has made FT pieces useful to FE builders, but the

families should remain distinct in a historical account.

## Why the FE mattered

The FE placed Ford at the centre of American performance during the

1960s. Its ordinary forms supplied the effortless torque expected in a

Thunderbird, Galaxie, or pickup. Its competition forms gave Ford the

cross-bolted mains, side-oiling system, large ports, strong bottom end,

and displacement needed to challenge Chrysler in NASCAR, Chevrolet in

drag racing, and Ferrari at Le Mans.

Its evolution also demonstrates that the most exotic solution is not

always the most successful. The 427 SOHC was astonishing but politically

excluded from its intended arena. The high-riser and Tunnel Port chased

airflow at the expense of packaging and flexibility. The 427 itself was

expensive to manufacture. The comparatively conventional 428 Cobra Jet

succeeded because it combined adequate breathing, abundant torque,

production feasibility, and durability at a price Ford could offer

broadly.

The Small Block Ford was Ford's adaptable lightweight athlete. The FE

was its heavyweight counterpart: physically substantial, mechanically

distinctive, and capable of assuming personalities that ranged from

nearly silent luxury-car torque to the thunder of a dual-quad side-oiler

at Le Mans. Its production life lasted less than two decades, but few

engine families packed more engineering ambition, racing history, and

mythology into their time.

## Sources and further reading

• Christ, S. (1983). How to rebuild big-block Ford engines.

HPBooks.

• Clarke, R. M. (Ed.). (1990). Musclecar & Hi-Po engines: Ford

big block. Brooklands Books.

• Ford Motor Company. (1958–1976). Ford, Edsel, Mercury,

Thunderbird, and truck sales literature and shop manuals. Ford

Heritage Vault. https://fordheritagevault.com/

• Ford Performance Parts. (n.d.). FE and 427 technical

information. https://performanceparts.ford.com/

• 428 Cobra Jet Registry. (n.d.). 428 Cobra Jet engine

specifications. https://www.428cobrajet.org/

• Reid, G. (2013). How to rebuild the big-block Ford.

CarTech.

• SEMA. (n.d.). Full power treatment. https://www.sema.org/

Specification note: Ford issued numerous engine codes and revised

ratings across vehicle lines, transmissions, model years, and emissions

jurisdictions. The figures above are representative United States

factory specifications. Pre-1972 horsepower and torque figures are SAE

gross; later figures are SAE net unless otherwise stated. Racing engines

frequently exceeded their published production ratings.