# The Small Block Ford

Published October 23, 2026

Topics: Automobiles, Ford, Engines, Motor Racing

In the early 1960s, Ford needed a modern V-8 that was smaller and

lighter than its Y-block, yet adaptable enough to power everything from

compact sedans to racing cars. The result was the engine enthusiasts now

call the Small Block Ford or Windsor V-8. It began with only 221 cubic

inches and 145 advertised horsepower, grew through 260, 289, 302, and

351 cubic inches, survived the emissions era, acquired electronic fuel

injection, and remained in production through the 2001 model year.

Few engines have had a wider life. Variants powered Fairlanes,

Falcons, Mustangs, Cobras, Sunbeam Tigers, pickups, vans, Explorers, and

full-sized Fords and Mercurys. Racing versions won in Trans-Am,

conquered Le Mans in the Ford GT40, transformed Indianapolis first with

pushrods and then with four camshafts and turbocharging, and turned the

Shelby GT350 and Boss 302 into permanent landmarks. The engine's

identity did not come from any single displacement or performance level.

It came from a compact, lightweight architecture that Ford could revise

repeatedly without losing the basic virtues of the original design.

## What “Small Block Ford” means

The term normally refers to the Windsor-family pushrod V-8 introduced

for the 1962 Fairlane. All versions used a cast-iron block, a single

camshaft in the block, two overhead valves per cylinder operated by

pushrods and rocker arms, and 4.380-inch bore spacing. The 221, 260,

289, 302, and 255 shared a low deck of approximately 8.206 inches. The

351W retained the architecture and 4.000-inch bore but used a taller

deck—9.480 inches initially and approximately 9.503 inches later—to

accommodate its 3.500-inch stroke.

“Windsor” is useful but imperfect shorthand. Early engines were

manufactured primarily at Ford's Cleveland Engine Plant, not merely in

Windsor, Ontario. More important, the 351 Windsor and 351 Cleveland are

not versions of the same block. The 351C belongs to Ford's later

335-series family, with a different block, different cooling

arrangement, and canted-valve cylinder heads. The Boss 302 is a

deliberate hybrid: a special Windsor-family 302 block fitted with heads

derived from the forthcoming 351 Cleveland.

Ford's later 5.0-litre pushrod V-8 is the 302 under a metric name.

Its actual displacement is approximately 4.95 litres, but “5.0” was far

more memorable—and eventually became a performance identity of its own.

The modern 5.0-litre Coyote, introduced for the 2011 Mustang, is an

overhead-cam Modular-family engine, not a continuation of the original

Small Block Ford.

## The family dimensions

Version

Principal model years

Bore

Stroke

Nominal displacement

Basic distinction

221

1962–1963

3.500 in.

2.870 in.

221 cu. in./3.6 L

Original Fairlane V-8

260

1962–1964

3.800 in.

2.870 in.

260 cu. in./4.3 L

Larger bore in the original block

289

1963–1968

4.000 in.

2.870 in.

289 cu. in./4.7 L

Established the family's standard 4.000-inch bore

302/5.0

1968–2001

4.000 in.

3.000 in.

302 cu. in./4.9 L

Longer stroke; the longest-lived version

255/4.2

1980–1982

3.680 in.

3.000 in.

255 cu. in./4.2 L

Economy-era small-bore derivative of the 302

351W/5.8

1969–1996

4.000 in.

3.500 in.

351 cu. in./5.8 L

Taller-deck, longer-stroke Windsor

The dimensions reveal Ford's development strategy. The 221, 260, and

289 shared the same 2.870-inch stroke, with displacement added through

progressively larger bores. The 302 retained the 289's 4.000-inch bore

and added a 3.000-inch stroke. The 351W kept that bore, raised the deck,

and increased stroke to 3.500 inches. Only the fuel-economy-minded 255

reversed the pattern, combining the 302 stroke with a smaller 3.680-inch

bore.

## Model-year progression at a

glance

Ford offered different calibrations by vehicle line, transmission,

emissions jurisdiction, and intended use, so no short table can list

every engine code. This timeline shows the principal United States

passenger-car and light-truck stages.

Model years

Principal Small Block Ford varieties

1962

221 2V; 260 2V introduced during the model year; special

high-performance 260

1963

221 and 260 2V; 289 2V introduced; 289 High Performance introduced

for the Fairlane; racing-only 255 pushrod Indy engine

1964

260 2V; 289 2V, D-code 4V, and K-code High Performance; 260 offered

in the early Mustang; racing-only 255 DOHC Indy engine

1965–1967

289 C-code 2V, A-code 4V, and K-code High Performance; Shelby GT350

versions above the standard Ford ratings

1968

289 2V continued briefly; 302 2V and 4V replaced it across most Ford

lines; racing-only 302 Tunnel Port appeared in Trans-Am; turbocharged

DOHC Indy development began

1969–1970

Standard 302; Boss 302; 351W 2V and 4V; turbocharged DOHC Indy

engine; the 351 Cleveland began replacing the 351W in many

performance-car applications during 1970

1971–1977

Low-compression 302s in passenger cars and light trucks; 351W

increasingly concentrated in larger cars, trucks, vans, and marine

applications

1978–1979

The 302 increasingly marketed as the 5.0 litre; carburetion remained

standard

1980–1982

Economy-oriented 255/4.2 alongside selected 302/5.0 applications;

5.0 H.O. returned to the Mustang for 1982

1983–1985

Four-barrel 5.0 H.O. with manual transmission; CFI on selected

automatic-transmission cars; roller camshaft added for 1985

1986–1988

Sequentially injected Mustang 5.0 H.O.; truck 5.0 and 5.8

calibrations; mass-air metering began in California for 1988

1989–1993

Mass-air 5.0 H.O. in all United States Mustangs; GT40-equipped 1993

Cobra; injected 5.0 and 5.8 truck engines

1994–1995

Final pushrod 5.0 Mustang GT and Cobra; 351W in the 1995 Cobra R;

5.8 EFI continued in trucks and vans

1996–2001

Mustang changed to the Modular V-8; 5.0 production continued in the

Explorer and Mountaineer through the 2001 model year

## 1962–1963: the

221 establishes the architecture

The 221-cubic-inch Fairlane V-8 debuted for the 1962 model year. Its

thin-wall casting made it unusually light and compact for an American

V-8—roughly 200 pounds lighter than some contemporary large engines. It

had a 3.500-inch bore, a 2.870-inch stroke, an 8.7:1 compression ratio,

and a two-barrel carburetor. Ford rated it at 145 horsepower at 4,400

rpm and 216 pound-feet at 2,200 rpm, under the SAE gross system then in

use.

Those numbers were modest, but the package mattered more than the

output. The little V-8 fit Ford's new intermediate and compact cars

without overwhelming their front suspensions or engine compartments. It

established the low-deck block, five-main-bearing crankshaft, separate

front cover, and lightweight construction that would define the family.

Ford built approximately 371,000 before ending production during 1963,

after buyers showed a clear preference for its larger descendants.

## 1962–1964: the 260 adds

usable torque

The 260 Challenger V-8 appeared during the 1962 model year. Ford

enlarged the bore to 3.800 inches while retaining the 221's 2.870-inch

stroke. With 8.8:1 compression and a two-barrel carburetor, the standard

engine was rated at 164 gross horsepower at 4,400 rpm and approximately

258 pound-feet at 2,200 rpm.

The 260 spread quickly from the Fairlane and Mercury Meteor to the

Falcon, Comet, full-sized Ford, and early 1964½ Mustang. It also

demonstrated how far the lightweight architecture could travel. Carroll

Shelby used a specially prepared, four-barrel 260 in the earliest

Cobras, and the Sunbeam Tiger placed the compact Ford V-8 in a British

sports car originally designed around a four-cylinder engine.

A small number of high-performance 260s received a four-barrel

carburetor, a hotter camshaft, stronger valve springs and connecting

rods, and higher compression. Ford rated the Shelby-oriented version at

approximately 260 gross horsepower. The standard 260 disappeared after

1964 because the 289 offered more performance with almost no packaging

penalty.

## 1963–1968:

the 289 becomes the classic Small Block Ford

The 289 arrived during 1963. Its 4.000-inch bore became the family's

enduring standard, while the short 2.870-inch stroke made the engine

notably oversquare and willing to rev. Ford offered several distinct

personalities under the same displacement.

Representative version

Model years

Induction

Compression

Advertised power

Advertised torque

Base 289

1963–1964

2-barrel

8.7:1

195 hp @ 4,400 rpm

258 lb.-ft. @ 2,200 rpm

D-code 289

1964

4-barrel

9.0:1

210 hp @ 4,400 rpm

Approximately 300 lb.-ft.

C-code 289

1965–1967

2-barrel

9.3:1

200 hp @ 4,400 rpm

282 lb.-ft. @ 2,400 rpm

A-code 289

1965–1967

4-barrel

10.0:1

225 hp @ 4,800 rpm

305 lb.-ft. @ 3,200 rpm

K-code High Performance

1963–1967

4-barrel

10.5:1

271 hp @ 6,000 rpm

312 lb.-ft. @ 3,400 rpm

Shelby GT350

1965–1966

Special 4-barrel

10.5:1

306 hp @ 6,000 rpm

329 lb.-ft. @ 4,200 rpm

All figures in this table are SAE gross ratings, and specifications

varied somewhat by application and model year. The differences were much

more substantial than carburetor size alone. The 271-horsepower K-code

used a solid-lifter camshaft, stronger connecting rods and rod bolts,

revised main-bearing caps, screw-in rocker studs, dual-point ignition,

freer-flowing exhaust manifolds, and careful assembly. It was not merely

a standard 289 wearing a four-barrel carburetor.

Shelby raised the performance level again for the GT350 with an

aluminium high-rise intake, a larger Holley four-barrel, tubular exhaust

headers, and other changes. Racing 289s powered Cobras, Shelby Daytona

Coupes, early GT40s, and championship-winning Trans-Am Mustangs. The

production 289's great achievement was breadth: it could be an

economical two-barrel family-car engine, a lively four-barrel Mustang

option, or the foundation of an internationally successful racing

engine.

## 1963–1971: Indy interlude

The Indianapolis engines require a qualification. They belonged to

the Small Block Ford story because Ford developed them from the Fairlane

V-8's architecture, bore spacing, and basic block patterns, but they

were purpose-built racing engines, not showroom varieties that happened

to receive special cylinder heads. Each step moved farther from the

production engine: the first retained pushrods, the second replaced the

entire valvetrain with four overhead camshafts and 32 valves, and the

third added a turbocharger and dramatically reduced displacement.

Indy version

Principal years

Bore × stroke

Induction

Compression

Representative output

255 pushrod

1963

3.760 × 2.870 in.

Four Weber carburetors

Approximately 12.5:1

365–376 hp at approximately 7,200 rpm;

peak torque not consistently published

255 DOHC

1964–1968

3.760 × 2.870 in.

Hilborn mechanical fuel injection

Approximately 12.5:1

425 hp at 8,000 rpm; 295 lb.-ft. at 6,400

rpm

159 turbo DOHC

1968–1970 factory development; later

Ford-derived and Foyt-badged versions

Approximately 3.500 × 2.070 in. in its

159-cu.-in. form

Mechanical fuel injection and a single turbocharger

Configuration- and boost-dependent

Approximately 700–750 hp by 1969; later

versions approached or exceeded 800 hp

These are representative racing specifications, rather than

homologated production ratings. Ford revised the engines continually,

while USAC changed the displacement and boost rules. Turbocharged

output, torque, and effective cylinder pressure therefore varied with

the permitted boost, fuel, qualifying specification, and race

specification.

## 1963: the 255-cubic-inch

pushrod engine

Dan Gurney persuaded Ford and Colin Chapman that a light, mid-engined

Lotus powered by a production-derived American V-8 could challenge the

heavy, front-engined Offenhauser roadsters at Indianapolis. Ford began

with the 260-cubic-inch Fairlane small-block, reduced the racing engine

to 255 cubic inches with a 3.760-inch bore and the production family's

2.870-inch stroke, and retained a single camshaft, pushrods, and two

valves per cylinder.

Almost everything around that familiar arrangement was specialized.

Ford used modified patterns to cast the block and cylinder heads in

aluminium, fitted four-bolt main-bearing caps, installed a magnesium

sump and dry-sump lubrication, drove the camshaft by gears, raised

compression to approximately 12.5:1, and fed the engine through four

58-mm Weber carburetors. On 103-octane gasoline, it produced

approximately 365–376 horsepower at about 7,200 rpm. Because gasoline

required fewer fuel stops than the methanol-burning Offenhausers,

efficiency was part of the race strategy, not merely a laboratory

virtue.

The engine made its Indianapolis 500 debut in the Lotus 29 on May 30,

1963. Jim Clark finished second, Dan Gurney finished seventh, and the

low, light, mid-engined cars demonstrated that the traditional

roadster's reign was nearly over. Clark then won the Tony Bettenhausen

200 at Milwaukee on August 18, 1963, giving the pushrod Lotus-Ford its

major USAC victory. The engine lasted only one season as Ford's

front-line Indianapolis design, but it proved both the chassis concept

and the small-block foundation.

## 1964–1968: the

naturally aspirated DOHC engine

For 1964, Ford retained the Fairlane-derived aluminium block, the

3.760-inch bore, the 2.870-inch stroke, and the 255-cubic-inch

displacement, but replaced the pushrod top end completely. New aluminium

cylinder heads carried two overhead camshafts per bank and four valves

per cylinder around a central spark plug. A train of 14 straight-cut

gears drove the camshafts, water pump, oil pumps, and alternator. The

engine also reversed the usual port arrangement: individual intake

trumpets stood outside the cylinder banks, while the exhaust ports

opened into the valley. Hilborn mechanical fuel injection replaced the

four Webers.

The four-cam engine made approximately 425 horsepower at 8,000 rpm

and 295 pound-feet at 6,400 rpm on gasoline, while preserving the

fuel-mileage advantage Ford had sought in 1963. It was a profound

transformation: the same basic lower-engine ancestry now supported a

pent-roof combustion chamber, 32 valves, direct-acting bucket tappets,

and engine speeds that a production 260 could never have tolerated.

Jim Clark put the Lotus 34 on the pole for the Indianapolis 500 on

May 30, 1964, although both Team Lotus cars retired. The developed

combination achieved its purpose one year later. Clark dominated the

Indianapolis 500 on May 31, 1965, in the Lotus 38-Ford, giving Ford its

first victory in the race and Indianapolis its first mid-engined winner.

Graham Hill won on May 30, 1966, in a Lola T90-Ford, and A. J. Foyt won

on May 30, 1967, in his Coyote-Ford. Three successive victories in three

different chassis showed that the four-cam Ford was more than a Lotus

curiosity; it had become the benchmark American championship-car

engine.

## 1968 onward: the

turbocharged DOHC engine

By 1968, turbocharged Offenhausers had erased the naturally aspirated

Ford's power advantage. Ford's answer kept the four-cam, 32-valve

architecture, mechanical injection, aluminium construction, dry-sump

lubrication, and gear-driven camshafts, but added a single Garrett

turbocharger. The forced-induction displacement allowance was 168 cubic

inches in 1968 and 159 cubic inches from 1969, so Ford reduced both bore

and stroke; a representative 159-cubic-inch version used an

approximately 3.500-inch bore and 2.070-inch stroke.

The smaller engine produced roughly 700–750 horsepower by 1969, with

the exact figure governed as much by boost rules and durability

requirements as by hardware. That was nearly twice the output of the

1963 pushrod engine from barely five-eighths of its displacement. The

price was heat, plumbing complexity, throttle lag, and immense

mechanical stress.

Mario Andretti won the Indianapolis 500 on May 30, 1969, in a

Hawk-Ford, and Al Unser won on May 30, 1970, and May 29, 1971, in turbo

Ford-powered Colts. Ford withdrew direct factory support after the 1970

season, and A. J. Foyt acquired the program and continued developing the

engine under his own name. The line's final Indianapolis victory came

when Foyt won on May 29, 1977, in a Coyote fitted with the turbocharged

Ford-derived, Foyt-badged V-8.

Taken together, the three versions trace an extraordinary engineering

progression. In six years, Ford advanced from a carbureted,

production-derived pushrod V-8 to a mechanically injected, 32-valve DOHC

engine, and then to a 159-cubic-inch turbocharged unit producing several

hundred additional horsepower. The production Windsor never became a

four-cam engine, but its compact proportions and robust bottom-end

concept gave Ford the starting point for one of Indianapolis's most

important engine families.

## 1968 onward:

three inches of stroke create the 302

For 1968, Ford increased stroke from 2.870 to 3.000 inches while

retaining the 4.000-inch bore. The resulting 301.6-cubic-inch engine was

marketed as the 302. The ordinary version used hydraulic lifters and

two-bolt main-bearing caps. A representative 1968 two-barrel engine had

9.5:1 compression and was rated at approximately 210 gross horsepower

and 295 pound-feet; a four-barrel version was rated at approximately 230

gross horsepower.

The extra stroke gave the 302 more low- and mid-range torque than the

289 without changing its external dimensions significantly. It became

Ford's standard compact V-8, serving in Mustangs, Mavericks, Torinos,

intermediates, full-sized cars, trucks, vans, sport-utility vehicles,

and marine applications. Ford began calling it a 5.0-litre engine during

the late 1970s, although its exact metric displacement rounded closer to

4.9 litres.

The ordinary 302's published output declined sharply during the early

1970s. Compression fell, cam timing and carburetion were softened,

exhaust systems became more restrictive, and emissions equipment

increased. Just as importantly, the industry changed its measuring

system.

## Gross horsepower versus

net horsepower

Before 1972, American manufacturers normally advertised SAE gross

horsepower measured on a test stand without a production air cleaner,

exhaust system, cooling fan, alternator, power-steering pump, or other

accessories. SAE net ratings measured a production-configured engine

with its normal intake, exhaust, and accessories. Net figures were lower

even when the physical engine changed little.

Consequently, a fall from approximately 210 gross horsepower for an

early 302 to roughly 140 net horsepower in 1972 does not mean that 70

usable horsepower disappeared in one year. Some performance was

genuinely lost to lower compression, leaner calibration, retarded

timing, and emissions controls, but a substantial portion of the

apparent fall came from the new rating method. Comparisons within the

gross era or within the net era are useful; direct comparisons across

1971–1972 require caution.

By the middle 1970s, some passenger-car 302s were rated near 122 net

horsepower. They remained durable, smooth, and torquey enough for

ordinary service, but the performance identity built by the 289 had

largely disappeared. It would return under the 5.0 H.O. name.

## 1968: the 302 Tunnel

Port aims too high

Between the K-code 289 and the Boss 302 came one of Ford's most

ambitious—and least successful—racing small-blocks. Chevrolet's new

302-powered Camaro Z/28 had ended the 1967 Trans-Am season with

consecutive victories, making clear that Ford's successful 289 and

GT40-style heads would not retain their advantage indefinitely. For

1968, Ford combined a heavy-duty, four-bolt-main 302 block derived from

its GT40 programme with radically different Tunnel Port cylinder

heads.

The concept came from Ford's successful 427 FE Tunnel Port. A

conventional intake port must curve around the pushrod, limiting its

size and forcing a compromise in shape. Ford instead created enormous,

nearly straight, 2.2-inch-diameter intake passages and ran the pushrods

through thin metal tubes inside the ports. The tubes gave the engine its

name: the intake charge flowed around the pushrod tunnels rather than

around an obstruction cast into the side of each port. The arrangement

permitted large valves, very high airflow, a dual-quad aluminium intake

manifold, and extraordinary high-rpm power.

On an engine dynamometer, a properly prepared racing Tunnel Port

could produce well over 400 horsepower and operate beyond 8,000 rpm.

Shelby's Trans-Am Mustangs commonly used 8,500-rpm shift points and

occasionally reached 9,000 rpm. Those figures were remarkable for a

302-cubic-inch pushrod V-8 in 1968, but they also identify the central

problem. The huge, straight ports moved air too slowly at lower engine

speeds, sacrificing torque and throttle response coming out of corners.

The narrow power band suited an oval-track engine better than a

road-racing engine that repeatedly accelerated from different

speeds.

Reliability was worse. The reciprocating assembly, valvetrain, oiling

system, and GT40-based block were all being asked to survive engine

speeds roughly 1,000–1,500 rpm higher than the previous Trans-Am 289.

Jerry Titus and Ronnie Bucknum gave the Tunnel Port a brilliant debut by

winning the Trans-Am class in the 1968 24 Hours of Daytona, but the rest

of the season was marked by repeated engine failures. At Continental

Divide, the Shelby team reportedly lost six engines over the race

weekend. Shelby-prepared engines won two of their four races, while the

later Ford-built engines won only once in nine events. Penske's Camaros

and Mark Donohue took the championship decisively.

Ford had planned a milder street version to satisfy SCCA homologation

requirements. Contemporary descriptions specified two Autolite

four-barrel carburetors, 10.5:1 compression, smaller 2.01-inch intake

and 1.53-inch exhaust valves, a hydraulic camshaft, cast-iron exhaust

manifolds, and emissions equipment. Ford proposed an implausibly modest

rating of 240 gross horsepower, but no production Mustang ever received

the engine. A 12.5:1-compression prototype tested by Car and

Driver may have been the only street-legal Tunnel Port Mustang Ford

completed.

The Tunnel Port was not a foolish engine; it was an engine optimized

around the wrong goal. Ford pursued the largest peak horsepower number

when Trans-Am required a broad torque curve and endurance. Even its

airflow lesson proved valuable. The Boss 302's canted-valve heads also

had very large ports, but their passages narrowed and then expanded

toward the valve, increasing mixture velocity—the Venturi principle that

the Tunnel Port's nearly constant-area “sewer pipes” lacked. Ford

abandoned the Tunnel Port after a single season and redirected its

effort to the more producible, more flexible Boss 302.

## 1969–1970: the

Boss 302 combines two Ford families

Ford created the Boss 302 to defeat Chevrolet's Camaro Z/28 in the

SCCA Trans-American Sedan Championship. The engine retained the

Windsor-family 4.000-inch bore and 3.000-inch stroke, but it was far

more specialized than the production 302. It used a high-strength block

with four-bolt centre main caps, a forged steel crankshaft, heavy-duty

connecting rods, solid lifters, a four-barrel Holley carburetor, and

enormous canted-valve cylinder heads developed for the forthcoming 351

Cleveland.

The Boss heads flowed exceptionally well at high speed—arguably too

well for docile low-speed street operation. Ford used 10.5:1 compression

and advertised 290 gross horsepower at 5,800 rpm and 290 pound-feet at

4,300 rpm. Contemporary racers and later dynamometer testing have

encouraged decades of debate over whether the engine was deliberately

underrated, but the official figure is the appropriate historical

benchmark.

The 1969 Boss 302 Mustang made the engine eligible for Trans-Am, and

the improved 1970 combination delivered the manufacturers' championship

to Ford. The Boss was produced for only two model years, yet it

permanently linked the Small Block Ford with high-rpm road racing. It

also created the familiar “Clevor” idea: Windsor-family block below,

Cleveland-style heads above.

## 1969–1996: the tall-deck 351

Windsor

Ford introduced the 351W for 1969. Its 4.000-inch bore matched the

289 and 302, but a 3.500-inch stroke required a taller block. The deck

height rose from roughly 8.206 inches to 9.480 inches, later

approximately 9.503 inches. The wider intake manifold, longer connecting

rods, larger main bearings, and increased block height mean that a 351W

is not simply a stroked 302 assembled from interchangeable parts.

The original 1969 two-barrel version used approximately 9.5:1

compression and produced 250 gross horsepower at 4,600 rpm and 355

pound-feet at 2,600 rpm. The four-barrel version raised compression to

approximately 10.7:1 and was rated at 290 gross horsepower at 4,800 rpm

and 385 pound-feet at 3,200 rpm. It gave Mustangs, Cougars, Fairlanes,

and Torinos the torque of a larger engine without the mass of an

FE-series big-block.

Representative 351W era

Induction

Compression

Advertised output

Character

1969 2V

2-barrel

Approximately 9.5:1

250 gross hp/355 lb.-ft.

Strong, flexible passenger-car engine

1969 4V

4-barrel

Approximately 10.7:1

290 gross hp/385 lb.-ft.

High-compression performance version

1972 passenger car

2-barrel

Approximately 8.3:1

About 153 net hp

Emissions-era, regular-fuel calibration

Late-1980s truck/van

4-barrel, then EFI

Approximately 8.3:1

Application-dependent

Torque-oriented 5.8-litre service

1993–1995 F-150 Lightning

Multiport EFI

Approximately 8.8:1

240 net hp/340 lb.-ft.

GT40 induction and heads

1995 Mustang Cobra R

Multiport EFI

Approximately 9.0:1

300 net hp/365 lb.-ft.

Limited-production road-racing homologation special

The Windsor 351 was gradually displaced in passenger cars by the 351

Cleveland and then concentrated in trucks, vans, police vehicles, marine

service, and performance specials. A four-barrel truck version returned

during the 1980s; electronic fuel injection followed for 1988. Roller

lifters arrived late in production. The first-generation F-150 Lightning

and the 1995 Mustang Cobra R demonstrated that the old architecture

could still deliver serious performance three decades after its

introduction.

## The Cleveland clarification

Ford's overlapping use of “351” created lasting confusion. The 351

Windsor, 351 Cleveland, 351M, and 400 are not interchangeable versions

of one engine.

• The 351W is the tall-deck member of the original

Small Block Ford family.

• The 351C, introduced for 1970, belongs to the 335

series. It has a different block, dry intake manifold, and canted-valve

heads with either moderate 2V or enormous 4V ports.

• The 351M and 400 are taller-deck 335-series engines

intended primarily for larger cars and trucks.

• The Boss 302 combines a special Windsor-family 302

bottom end with Cleveland-derived canted-valve heads.

The Cleveland is commonly called a Ford small-block in the broad

sense that it is physically smaller than the FE and 385-series

big-blocks. In the narrower enthusiast and parts-catalogue sense,

however, “Small Block Ford” or “SBF” generally means the Windsor

family.

## 1980–1982: the 255

reaches the low point

Ford expected fuel-economy requirements to end the 302's

passenger-car career, so it created the 255 for 1980. The engine

retained the 302's 3.000-inch stroke but reduced the bore to 3.680

inches. Small-port heads, small valves, mild cam timing, low

compression, and two-barrel or variable-venturi carburetion emphasized

economy rather than performance.

Depending on application and year, factory ratings were generally

around 115–122 net horsepower. The 255 appeared in Mustangs, Capris,

Fairmonts, Thunderbirds, LTDs, and other Ford and Mercury models, but it

delivered little fuel-economy benefit over the 302 and considerably less

performance. Ford discontinued it after 1982. Its brief life showed the

danger of shrinking bore in an engine whose cylinder-head breathing and

valve space depended on the 4.000-inch dimension.

## 1982–1995: the 5.0

H.O. restores the reputation

The 302 returned to the Mustang as the 5.0 High Output for 1982. At

first, the revival was modest: a two-barrel carburetor, 157 net

horsepower, and 240 pound-feet. A four-barrel Holley arrived for

manual-transmission cars in 1983. Ford then added a roller camshaft,

better cylinder heads, freer intake and exhaust systems, and

increasingly sophisticated electronic control.

Mustang model year

Induction

Compression

Power

Torque

Principal development

1982

2-barrel carburetor

Approximately 8.4:1

157 hp

240 lb.-ft.

H.O. name returns

1983–1984 manual

4-barrel Holley

Approximately 8.4:1

175 hp

245 lb.-ft.

Four-barrel performance restored

1984–1985 automatic

Central fuel injection

Approximately 8.4:1

165–180 hp

Approximately 245–260 lb.-ft.

Early electronic injection, calibrated for the AOD

1985 manual

4-barrel Holley

Approximately 8.4:1

210 hp

265 lb.-ft.

Roller cam and revised heads; final carbureted Mustang V-8

1986

Sequential EFI

9.2:1

200 hp

285 lb.-ft.

First sequentially injected 5.0 Mustang

1987–1992

Sequential EFI

Approximately 9.0:1

225 hp

300 lb.-ft.

E7TE heads, revised intake, and mass-air metering phased in

1993 Mustang GT

Sequential EFI

Approximately 9.0:1

205 hp

275 lb.-ft.

Ford adopted a more conservative rating

1993 Mustang Cobra

Sequential EFI

Approximately 9.0:1

235 hp

280 lb.-ft.

GT40 heads and intake

1994–1995 Mustang GT

Sequential EFI

Approximately 9.0:1

215 hp

285 lb.-ft.

Revised SN95 installation

1994–1995 Mustang Cobra

Sequential EFI

Approximately 9.0:1

240 hp

285 lb.-ft.

GT40-based performance package

Electronic fuel injection changed the engine's character without

changing its fundamentals. The 1986 Mustang used speed-density

sequential injection; mass-air metering reached California cars in 1988

and all United States Mustangs for 1989. Mass air measured incoming

airflow directly and tolerated engine modifications better than the

speed-density strategy, helping create the enormous Fox-body

aftermarket.

The 1987–1992 5.0 H.O.'s 225 horsepower may seem mild today, but the

Mustang was relatively light, its 300 pound-feet arrived early, and

inexpensive modifications produced substantial gains. The engine made

performance accessible again. More importantly, it restored the direct

line from an ordinary production Mustang to weekend drag racing,

autocross, road racing, and home engine building.

## 1996–2001:

the Explorer closes the production story

The Mustang replaced its pushrod 5.0 with the overhead-cam 4.6-litre

Modular V-8 for 1996, but the original Small Block Ford did not die

immediately. Ford continued the 5.0 in the Explorer and Mercury

Mountaineer through the 2001 model year. These engines used electronic

fuel injection, a roller camshaft, distributorless ignition in later

applications, and efficient GT40 or GT40P-style cylinder heads and

intake manifolds. Typical ratings were approximately 210 net horsepower

and 280 pound-feet.

The final production form was far removed from the carbureted 221 of

1962, yet the family resemblance was unmistakable: the same bore

spacing, pushrod valvetrain, compact external dimensions, and

fundamental block layout. Nearly four decades of production made parts

plentiful and interchange possibilities almost endless—provided the

builder understood the important differences among balance factors,

firing orders, deck heights, oil-pan arrangements, accessory drives, and

fuel systems.

## Carburetion to

electronic fuel injection

The family also charts the history of American fuel delivery. The

first 221 used a simple two-barrel carburetor. Performance 260s, 289s,

302s, and 351Ws added four-barrel carburetors, which opened secondary

throttle bores when greater airflow was needed. During the emissions

era, increasingly complex vacuum controls and variable-venturi designs

tried to make carburetors meet demands they had never been designed to

satisfy.

Central fuel injection, or CFI, appeared on some early-1980s

applications as an electronic bridge: injectors sprayed fuel above a

throttle body resembling a carburetor. Multiport electronic fuel

injection placed an injector near each intake valve, while sequential

EFI timed each injector to its cylinder's intake cycle. Speed-density

control inferred airflow from pressure, temperature, and engine-speed

data; mass-air control measured it. By the end, the 5.0 could start

cleanly in any weather, compensate for altitude and temperature, meet

emissions standards, and retain the broad torque that had always made

the small-block attractive.

## Why the Small Block Ford

mattered

The Small Block Ford succeeded because its original proportions were

right. It was light enough for compact cars, narrow enough for engine

swaps, strong enough for ordinary service, and adaptable enough for

racing. The 4.000-inch-bore versions had room for useful valves and

cylinder-head development. The short-deck 289 and 302 could rev, while

the tall-deck 351W supplied torque and room for still larger aftermarket

combinations.

Its history also mirrors the American automobile industry. The 221

and 260 answered the demand for lighter intermediates. The 289

flourished during the first Mustang and Cobra era. The Boss 302 was

shaped by Trans-Am homologation. Low-compression 302s and the

unfortunate 255 reflected fuel and emissions pressures. The injected 5.0

H.O. marked Detroit's performance recovery, and the Explorer version

carried the design into the twenty-first century.

Ford has built engines with greater power, more advanced materials,

and far more sophisticated valve control. Few have combined such a

modest beginning with such an expansive life. From a 145-horsepower

Fairlane to a GT350, a Le Mans-winning GT40, a Boss 302, a Fox-body 5.0,

and a Cobra R, the Small Block Ford repeatedly became what its moment

required without ceasing to be recognizably itself.

## Sources and further reading

• Ford Motor Company. (1962–2001). Ford vehicle specifications,

sales brochures, and shop manuals. Ford Heritage Vault. https://fordheritagevault.com/

• Ford Performance Parts. (n.d.). Boss 302 and small-block Ford

technical information. https://performanceparts.ford.com/

• Hot Rod. (1964/2024). Ford's 1964 Indy V8 would forever

change the look and sound of the Indy 500. https://www.hotrod.com/features/ford-1964-indy-500-v8-engine-hot-rod-magazine-march-1964

• Indianapolis Motor Speedway. (n.d.). Indianapolis 500 historical

race results. https://www.indianapolismotorspeedway.com/events/indy500/history/historical-stats/race-results

• Race Engine Technology. (2024, January 26). Special investigation:

Clean-sheet Indy race engines, 1940–89. https://www.raceenginetechnology.com/Suppliers/special-investigation-clean-sheet-indy-race-engines-1940-89

• English, E. (2013, May 15). The story behind Ford's ill-fated 1968

Tunnel-Port 302. Hot Rod. https://www.hotrod.com/news/hrdp-1305-the-story-behind-fords-iii-fated-1968-tunnel-port-302

• Hammill, D. (2011). How to power tune Ford V8. Veloce

Publishing.

• Martin, I. (2010). Ford Windsor small-block performance.

HPBooks.

• McLaughlin, P. (2010). How to build Ford Windsor small-block

engines. CarTech.

• Reid, G. (2015). Ford small-block engine parts interchange.

CarTech.

• Shelby American Automobile Club. (1997). The Shelby American

World Registry. SAAC.

Specification note: Ford offered numerous calibrations across

different car, truck, marine, export, and performance applications.

Horsepower, torque, compression ratio, carburetor, camshaft, and even

internal components can vary within a displacement and model year. The

tables identify representative United States factory specifications

rather than every individual engine code. Pre-1972 power figures are SAE

gross; later figures are SAE net unless otherwise stated.